# Plan for GPS Integration

## Step 1: Connect the GPS to the Raspberry Pi

The GPS module communicates with the Raspberry Pi through the UART serial interface.

Connect the GPS modules:

- VCC → 5V on the Raspberry Pi

- GND → Ground

- TX → GPIO15 (RX) on the Pi

- RX → GPIO14 (TX) on the Pi

# Step 2: Enable Serial Communication on the Raspberry Pi and Verify Data Transmission

# Step 3: Install GPS Parsing Libraries

Install Python libraries that will allow the Raspberry Pi to interpret the GPS messages.

Pyserial and Pynmea2 to collect the latitude and longitude from the data stream.

# Step 4: Create a GPS Data Reader Script

Write a Python program that continuously reads serial data from the GPS and extracts:

- Latitude

- Longitude

- Time

- Speed

# Step 6: Integrate GPS into the Robot’s Sensor System

The Raspberry Pi should now process data from:

- Camera: detects sidewalks or path edges

- LiDAR: detects nearby obstacles

- GPS: determines the robot’s global position

# Step 7: Define Target Destinations

Test GPS capabilities by defining a target location and collecting its GPS data.

- Read its current GPS coordinates.

- Compare them to the destination coordinates.

- Calculate the direction and distance to the target.

# Step 8: Combine GPS with Camera and LiDAR Logic and turn into a continuous loop

Create a main control loop that processes all sensor inputs.

- Read LiDAR data to detect obstacles.

- Process camera input to stay aligned with the sidewalk.

- Read and use GPS data to determine overall travel direction.

- Send movement commands to the motor controller.

*Will discuss method more in depth*